ZupControl

Operation Manual

# © 2002, wkl Lasertechnik

# Disclaimer

Although due care was taken in the preparation of this software and documentation, there is no warranty from wkl Lasertechnik or Lambda Electronics Inc. that this software will operate properly or in accordance with any specifications. Use of this software is expressly conditioned on agreement by the user that neither wkl Lasertechnik nor Lambda Electronics shall have any liability to the user or any other entity with respect to any liability, loss, or damage caused directly or indirectly from the use of or operation of this software.

ALL IMPLIED WARRANTIES, INCLUDING WITHOUT LIMITATION ANY IMPLIED WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, ARE EXPRESSLY DISCLAIMED.

For further questions regarding the ZERO UP series or any other questions please contact your LAMBDA USA contact address:

### LAMBDA ELECTRONICS INC.

3055 Del Sol Boulevard

San Diego, CA 92154

Tel : 619-575-4400

Fax: 619-575-7185

1-800-LAMBDA-4

www.lambdapower.com

# Introduction

This program “ZupControl“ supports real-time operation and controlling of one or two ZERO UP units of the manufacturer Nemic-Lambda Ltd.

These ZUP units will be connected on a free serial communication port, operation via  RS232 or RS485 interface of the PC (please refer to “ZUP User’s Manual“ pages 39/40 and pages 41/42). The baud rate is fixed of 9600bps to communicate with this software.

## Quick Start

- Choose “Options” to open Dialog box ”General options“ and set the number of your serial Interface (“Com port“). Additionally you have to decide how many units you will control with (“# of devices“, 1 or 2 is possible), then choose Apply.

- After decision on the number of connected units, choose “Options->Device1” (and “Options->Device 2“) to open Dialog box “Config device” and select the ZUP address (“Address“), the Model type (“Type“), as well as the programmable range for o/p voltage and current via the virtual knobs (“Dynamic voltage“ and “Dynamic current“), then choose Apply.

- To start the program choose “File->Run“ and the ZERO UP Units will control via this Virtual Front Panel on your PC.

# Menu Commands

File -> Run

The ZupControl session will start. After start sequence is complete the Virtual Front Panel is active, resulting in the menu commands  “File->Run“, “File->Exit“, “Options->General“, “Options->Device 1“, “Options->Device 2“ are locked as well as the “File->Stop“ released.

File -> Stop

The ZupControl session will stop.

File -> Exit

Exits the program.

Options -> General

Setting of general options.

Options -> Device 1

Settings of parameters for unit no. 1.

Options -> Device 2

Settings of parameters for unit no. 2.

Extras -> Show log

Lists commands which were made during active ZupControl session.

Extras -> Hide log

Closes Log window.

# Options

The options are selected in two levels. First as a general part and the other refer to an individual setting either for unit 1 or for both if device 1 and 2 are activated.

If you use the software for the first time it is necessary to configure the individual device first before you start the ZupControl session.

These settings are necessary only at first time to use this software. All parameters will be automatically stored in the Windows-Registry and so it will be still available after restarting the Program or at Windows-Newstart.

## General Options

Com-Port

Number of the serial interface to which the ZUP unit(s) is/are connected.

# of Devices

Number of connected units (1-2).

GUI autoupdate

Option, to readback the actual ratings of output –voltage and -current every XXXX ms from the connected ZUP units.

Allow external position of curr./volt. sliders

Option, to allow changing of the positions of the V and A adjustment knobs if we change any o/p voltage and current ratings.

Logging level

Advises what kind of messages will be displayed in the Log-window. (logs all messages, warnings and errors or only errors).

## Unit-Config device options

Name

Individual name of the connected unit. This name will be displayed on the Virtual Front Panel then.

Address

Sets the PSU address (1-31).

Type

Sets the connected Model Type.

Dynamic Current

If activated, the o/p current can be adjusted via the Virtual knob (and the +/- button) according the set programming range. If this is not done there is no adjustment possible!

Dynamic Voltage

If activated, the o/p voltage can be adjusted via the Virtual knob (and the +/- button) according to the set programming range. If this is not done there is no adjustment possible!

Execute init script

If activated, the choosed commands will send to the ZUP unit before each starting of a new control session sequence. This option is very helpful to reset the communication buffer of the internal status/error registers or to set the unit in any operational mode.

Execute init script -> DCL

Clears the communication buffer and the internal registers.

Execute init script -> set AST

Sets the auto-restart mode to On or Off.

Execute init script -> set FLD mode

Sets the Foldback protection to ON or Off. (Please refer to “user's manual“, page 44).

Execute init script -> set OVP

Sets the Over-Voltage-Protection levels in volts. Please refer to Over-voltage range settings given in the table 5-3 in the “user's manual“, page 44 for the individual models.

Execute init script -> set UVP

Sets the Under-Voltage-Protection limits in volts. Please refer to Under-voltage range settings given in the table 5-4 in the “user's manual“, page 45 for the individual models.

Execute init script -> set curr. to

Sets the output current in Amperes to the programmed Value.

Execute init script -> set volt. to

Sets the output voltage value in volts to the programmed voltage.

Execute init script -> set OUT

Sets the output to On or Off.

# The Virtual Front Panel

(1) Name of the connected unit (could be defined in the Config device option)

(2) Actual output current display

(3) Present programmed output current display

(4) Adjust Knob for output current (in defined o/p current range)

(5) Button to increase / decrease the o/p current in defined steps

(6) Actual output voltage display

(7) Present programmed output voltage display

(8) Adjust Knob for output voltage (in defined o/p voltage range)

(9) Button to increase / decrease the o/p voltage in defined steps

(10) The Operational Status Register records signals that are a part of the power supply’s normal operation.

(Please refer to “user’s manual”, page 45 for further details):

- OUT

- AST

- FLD

- CC

- CV

- ALM

(11) The Alarm Status/Error Codes - Register records fault conditions occurring during power supply operation and errors occurred during the programming of the PSU. (Please refer to “user’s manual”, page 46 for further details):

- OVP

- OTP

- A/C

- FLD

- PRG

- CMD

- OFL

- WCU

- WVO

(12) Power pushbutton that the power supply output is turned On and Off.